PART 1 GENERAL

1.01	SUMMARY

A.	Section Includes: This Section specifies design, supply and installation of window washing systems and suspended maintenance equipment.

1.02	RELATED SECTIONS

A.	Section [01 31 19 – Project Meetings]

B.	Section [01 61 00 – Common Product Requirements]

C.	Section [01 74 00 – Cleaning and Waste Management]

D.	Section [05 05 23 - Metal Fastenings: horizontal lifeline fasteners].

E.	Section [01 78 00 - Closeout Submittals].

1.03	REFERENCES

A.	American Institute of Steel Construction (AISC).

1.	AISC S342L [1993], Load and Resistance Factor Design Specification for Structural Steel Buildings (including Supplement No.1).

B.	Aluminum Association (AA).

1.	AA DAF 45, Designation System for Aluminum Finishes.

2.	AA ADM-1 [2000], Aluminum Design Manual.

D.	American Society of Mechanical Engineers (AMSE).

1.	ASME A120.1 [2006], Safety Requirements for Powered Platforms and Traveling Ladders and Gantries for Building Maintenance.

E.	American National Standards Institute / International Window Cleaning Association (ANSI/IWCA).

1.	ANSI/IWCA I-14.1 [2006], Window Cleaning Safety Standard.

F.	American Welding Society (AWS).

1.	AWS D1.2/D1.2M [2003], Structural Welding Code - Aluminum.

2.	AWS D1.1/D1.1M [2006], Structural Welding Code—Steel.

G.	ASTM International (ASTM).

1.	ASTM A123/A123M [2002], Standard Specification for Zinc (Hot-Dip Galvanized) Coatings on Iron and Steel Products.

2.	ASTM A167 [1999 (2004)], Specification for Stainless and Heat Resisting Chromium Nickel Steel Plate, Sheet and Strip.

3.	ASTM A276 [2006], Standard Specification for Stainless Steel Bars and Shapes.

4.	ASTM A492 [1995 (2004)] Standard Specification for Stainless Steel Rope.

5.	ASTM B221 [2006], Standard Specification for Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes.

H.	International Code Council (ICC).

1.	International Building Code.

I.	Occupational Safety and Health Administration (OSHA).

1.	OSHA 1910, Subpart D, Walking and Work Surfaces.

2.	OSHA 1910, Subpart F, Appendix C, Personal Fall Arrest Systems.

3.	OSHA Ruling on Window Cleaning by Bosun’s Chair.

4.	OSHA 1910.66 Subpart F, Powered Platforms.

1.04	ACTION SUBMITTALS

A.	General: Submit listed action submittals in accordance with Contract Conditions and Section [01 33 00 - Submittal Procedures].

B.	Shop Drawings: Indicate information on shop drawings as follows:

1.	Submit shop drawings showing complete layout and configuration of window cleaning and suspended maintenance system, including components and accessories.

2.	Indicate design and fabrication details, window "drops", hardware, and installation details.

3.	Include installation and rigging instructions and:

a.	Required restrictive working usage and general safety notes.

b.	Non-restrictive working usage and general safety notes.

4.	Ensure Shop Drawings are reviewed by Engineer licensed in State of D.C and submit [calculations] to [Architect] [Consultant] [Owner].

C.	Submit product data, including manufacturer’s technical data sheet, for specified products.

1.05	INFORMATION SUBMITTALS

A.	Quality Assurance:

1.	Test Reports: Certified test reports showing compliance with specified performance characteristics and physical properties.

2.	Certificates: Product certificates signed by manufacturer certifying materials comply with specified performance characteristics and criteria and physical requirements.

3.	Manufacturer’s installation instructions.

C.	Manufacturer’s field reports specified.

1.06	CLOSEOUT SUBMITTALS

A.	Submit 1-year standard manufacturer warranty documents specified.

B.	Operation and Maintenance Data: Submit Operation and Maintenance data for installed products in accordance with Section [01 78 00 - Closeout Submittals].

1.	Include:

a.	Manufacturer’s instructions covering maintenance requirements and parts catalog giving complete list of repair and replacement parts with cuts and identifying numbers.

b.	[1] copy of system Equipment Manual & Inspection Log Book, with "Initial Inspection - Certification for Use" and "Inspection Sign-Off" forms completed.

c.	[2] copies of reduced, “as-built shop drawing” showing equipment locations and details. Ensure drawing is posted adjacent exits to roof.

1.07	QUALITY ASSURANCE

A.	Qualifications:

1.	Installer experienced in performing work of this section who has specialized in installation of work similar to that required for this project.

2.	Manufacturer Qualifications: Manufacturer capable of providing field service representation during construction and approving application method.

B.	Regulatory Requirements.

1.	Comply with [Uniform Building Code (UBC)] [International Building Code (IBC)] [Building Code for [State] of D.C.

2.	Comply with OSHA regulations as follows:

a.	1910, Subpart D, Walking and Working Surfaces.

b.	Appendix C to 1910 Subpart F, Personal Fall Arrest Systems.

c.	OSHA Ruling on Window Cleaning by Bosun's Chair.

d.	1910.66, Subpart F, Powered Platforms.

C.	Pre-installation Meetings: Conduct pre-installation meeting to verify project requirements, manufacturer’s installation instructions and manufacturer’s warranty requirements. Comply with [Section 01 31 19 - Project Meetings].

1.08	DELIVERY, STORAGE AND HANDLING

A.	General: Comply with [01 61 00 - Common Product Requirements].

B.	Ordering: Comply with manufacturer’s ordering instructions and lead time requirements to avoid construction delays.

C.	Delivery:

1.	Deliver materials in manufacturer’s original packaging with identification labels intact and in sizes to suit project.

D.	Storage and Protection:

1.	Store materials protected from exposure to harmful weather conditions and at temperature conditions recommended by manufacturer.

E.	Transportation and Handling:

1.	Ensure center of gravity of davits weighing [80] lbs ([36.3] kg) or greater remains [36] inches ([915] mm) maximum above safe surface while transporting.

2.	Ensure davits requiring [80] lbs ([36.3] kg) or greater lifting effort are equipped with are equipped with mechanical means of hoisting into position.

F.	Packaging Waste Management:

1.	Separate waste materials for [reuse] [and] [recycling] [_____] in accordance with [Section 01 74 19 - Construction Waste Management and Disposal] [_____].

2.	Remove from site and dispose of packaging materials at appropriate recycling facilities.

3.	Collect and separate for disposal [paper] [plastic] [polystyrene] [corrugated cardboard] [_____] packaging material [in appropriate onsite bins] [_____] for recycling.

1.09	PROJECT AMBIENT CONDITIONS

A.	Installation Location: Assemble and erect components only when temperatures are above [40] degrees F ([4] degrees C).

1.10	SEQUENCING

A.	Sequence with other Work and Comply with window washing equipment manufacturer’s written recommendations for sequencing construction operations.

1.11	WARRANTY

A.	Project Warranty: Refer to Contract Conditions for additional project warranty provisions.

B.	Manufacturer’s Warranty: Submit, for Owner’s acceptance, manufacturer’s standard warranty document executed by authorized company official. Manufacturer’s warranty is in addition to, and does not limit, other rights Owner may have under Contract Documents.

C.	Warranty: Commencing on date of substantial completion set by [Owner] [Architect] [Consultant].

PART 2 PRODUCTS

2.01	MANUFACTURERS

A.	Ensure manufacturer has minimum [5] years experience in manufacturing window washing and suspended maintenance system components similar to or exceeding requirements of project.

B.	Manufacturer’s Insurance: Ensure manufacturer carries liability insurance to protect against product and system failure in amount of [five million dollars US ($5,000,000)] minimum.

2.02	PROPRIETARY PRODUCTS AND SYSTEMS

- Manufacturer: Pro-Bel Group of Companies, Phone: (905) 427-0616, USA Toll Free: (800) 461-0575, Fax: (905) 427-2545, E-mail: info@pro-belgroup.com, Internet URL : www.pro-bel.com

2.03	DESIGN PERFORMANCE REQUIREMENTS

A.	Design window cleaning and suspended maintenance system to suit project requirements to [AISC S342L] [and as indicated].

B.	Locate anchorages to suit suspension equipment specified.

C.	Design anchor components for cleaning and suspended maintenance equipment to [ASME A120.1].

1.	Ensure compatibility with industry standard equipment.

2.	Anchorage and anchor components: Designed by Engineer qualified in design of window cleaning and suspended maintenance equipment and licensed in State of D.C.

D.	Design system fall arrest safety anchors and equipment supports to [AISC S342L (including supplement No.1)] [and ANSI/IWCA I-14.1], and as follows:

1.	Comply with OSHA 1910, Subpart F, Appendix C.

2.	Supports for Suspended Platforms including davits:

a.	Safety factor against fracture or detachment: 4 to 1.

a.	Vertical service load: [1000] lbs ([4.45] kN) minimum.

b.	Rated load against fracture: [4000] lbs ([17.8] kN) minimum.

3.	Fall Arrest Safety Anchors:

a.	Fall arresting force safety factor of 2 to 1 without permanent deformation: [1800] lbs ([8.0] kN) minimum.

2.04	EQUIPMENT

A.	Anchors.

B.	Davits.

C.	Double Lanyard Horizontal Lifeline Systems.

D.	Monorail

2.05	ANCHORS

A.	Safety U-bars: [Stainless steel to ASTM A276,Type 304 with [35] Ksi ([240] MPa) minimum yield strength] [Mild steel, Type 300W with [44] Ksi ([300] MPa) minimum yield strength, hot-dip galvanized to ASTM A123/A123M].

1.	U-bar: [0.75] inches ([19] mm) minimum diameter material with [1.5] inches ([38] mm) eye opening.

B.	Hollow Steel Section (HSS) Piers: Mild steel, Type 300W with [50] Ksi ([350] MPa) minimum yield strength, [hot dipped galvanized to ASTM A123/A123M] [manufacturer’s polyurethane/polyurea coating system].

1.	Wall thickness [to suit application] [as indicated].

C.	Plate and other sections: Mild steel, Type 300W with [44] Ksi ([300] MPa) minimum yield strength, [hot dipped galvanized to ASTM A123/A123M] [manufacturer’s polyurethane/polyurea coating system].

1.	Wall thickness [to suit application] [as indicated].

D.	Seamless Spun Aluminum Flashing (for Roof Anchors): To AA ADM-1 Type 6061-T6 alloy and to ASTM B221.

1.	Deck flange flashing: [To NRCA Roofing and Waterproofing Manual recommendations] [In accordance with Section 07 62 00 Sheet Metal Flashing and Trim], [conformable mastic tape and torch applied heat-shrink rubber collar flashing] [detachable watertight stainless steel cap].

2.	Acceptable material: Pro-Bel Group, Aluminum Deck Flange Flashing.

E.	Miscellaneous Bolts, Nuts and Washers: [Mild steel, Type 300W with [44] Ksi ([300] MPa) minimum yield strength, hot-dip galvanized to ASTM A123/A123M] [Stainless steel to ASTM A276, Type 304 with [35] Ksi ([240] MPa) minimum yield strength.

2.06	DAVITS

A.	Davit Booms: [Ground] rigged, aluminum sections of engineered length and size [to suit application] [as indicated], equipped with: carrying handles; [stainless steel rolling trolley] [stainless steel friction trolley] [galvanized fixed shackle] on outboard end, designed to carry [1000] lbs ([4.5] kN) vertical service load, minimum.

1.	Ensure non-corrosive UV resistant data plate stating Maximum Service Capacity of boom, Manufacturer's Name, Serial No., Manufacturing Date, rated load and other pertinent information is prominently displayed.

B.	Davit Masts: Round tubular [aluminum][steel] section capable of rotating through 360° with carrying handles and connecting pins.

C.	Davit Arms: Davits to be demountable, portable, capable of being easily and quickly broken down into pieces weighing [80] [140] lbs ([36.3] [73] kg) maximum.

1.	Ensure davit arm booms equipped with rolling trolleys or friction trolleys have stops to prevent detachment from boom.

D.	Davit Bases: Round, mild steel, hollow section piers, Type 350W with [50] Ksi ([350] MPa) minimum yield strength, [hot dipped galvanized to ASTM A123/A123M] [manufacturer’s polyurethane/polyurea coating system] [with [0.75] inches ([19] mm) diameter U-bar safety anchor, and securement [to suit application] [as indicated].

E.	Acceptable Material: Pro-Bel Group, Davit System

2.15	DOUBLE LANYARD HORIZONTAL LIFELINE SYSTEM

A.	Stainless steel to ASTM A492, Type 316, [0.3125] inches ([8] mm) minimum diameter cable, [9127] lbs ([40] kN) minimum breaking strength with [permanently] [mechanically] swedged cable ends.

B.	Data plate: Ensure non-corrosive data plate stating Maximum Service Capacity of cable, Manufacturer's Name, Serial No., Manufacturing Date, rated load and other pertinent information is prominently displayed at cable system entry points.

C.	Tensioner: Stainless steel turnbuckle to ASTM A167, Type 316.

D.	Harness: Manufacturer's standard full body harness with double shock absorber lanyard.

E.	Acceptable Material: Pro-Bel Group, Double Lanyard Horizontal Lifeline System.

- Monorails

2.15	MONORAILS

A.	Monorails and Mounting: Aluminum extrusions to ASTM B221 [50] Ksi ([350] MPa) minimum yield strength and [65] Ksi ([450] MPa), minimum tensile strength and designed to carry [1000] lbs ([4.5] kN) minimum vertical service load.

B.	Monorail Finish:

1.	Exterior: Aluminum, to AA DAF-45

2.	Interior: include Mill Finish

C.	Capacity/Data Plates: Ensure non-corrosive data plate stating Maximum Service Capacity of monorail, Manufacturer's Name, Serial No., Manufacturing Date, rated load and other pertinent information is prominently displayed.

D.	Trolleys: Heavy-duty, Corrosion resistant, weatherproof rollers, [0.625] inches ([16] mm) minimum diameter stainless steel U-bar safety anchors to ASTM A276,Type 304 exterior finish.

1.	Interior finish to be powder coated mild steel to match monorail.

2.	Ensure trolleys to run freely under load with minimum discontinuity at rail splices.

3.	Provide removable end stops.

E.	Support Brackets: Cantilevered galvanized steel brackets as indicated.

1.	Locate brackets as close to and no greater than [16] inches ([400] mm) from monorail joints.

F.	Acceptable Materials: Pro-Bel Group, Monorail System.

2.20	ELECTRICAL REQUIREMENTS

A.	Receptacles: Waterproof, independently protected main line power receptacles rated 208 volts, 3 phase, 60 Hertz, 30 amperes.

1.	Acceptable material: Hubbell Twist-Lock.

B.	Ensure main line power is located [100] feet ([30] m) maximum from powered window washing equipment.

C.	Ensure power outlets experience 3% maximum voltage drop under full load.

D.	Cable Wall and Roof Anchors

1.	Acceptable material: Pro-Bel Group, Wall and Roof Anchors.

2.22	SOURCE QUALITY CONTROL

A.	Ensure window washing equipment components and materials are from single manufacturer.

2.23	PRODUCT SUBSTITUTIONS

A.	Substitutions: [In accordance with Section 01 25 13 - Product Substitution Procedures]] [No substitutions permitted].

PART 3 EXECUTION

3.01	INSTALLERS

A.	Provide experienced and qualified technicians to carry out erection, assembly and installation of window washing and suspended maintenance equipment system.

B.	Do steel welding to AWS D1.2/D1.2M.

C.	Do aluminum welding to AWS D1.1/D1.1M.

3.02	MANUFACTURERS INSTRUCTIONS

A.	Compliance: Comply with manufacturer’s written data, including product technical bulletins, product catalog installation instructions and Pro-Bel Group technical data sheets.

3.03	EXAMINATION

A.	Site Verification of Conditions:

1.	Verify that substrate conditions which have been previously installed under other sections or contracts, are acceptable for product installation in accordance with manufacturer’s instructions prior to installation of window washing equipment.

2.	Inform [Owner] [Architect] [Consultant] of unacceptable conditions immediately upon discovery.

3.	Proceed with installation only after unacceptable conditions have been remedied.

3.04	PREPARATION

A.	Ensure structure or substrate is adequate to support complete window washing equipment system.

B.	Ensure structural steel to receive safety anchors [has adequate bearing surface as indicated on shop drawings] [and] [has 100% welds between anchors and structural steel].

3.05	INSTALLATION

A.	Coordinate window washing equipment work with work of other trades, for proper time and sequence to avoid construction delays.

B.	Install window washing equipment plumb and level in accordance with manufacturer’s written instructions.

C.	[Mechanically fasten anchors in accordance with manufacturer’s recommendations] [Install anchors in concrete in accordance with Section 03 30 00 - Cast-in-Place Concrete and with manufacturer’s recommendations.

D.	Accurately fit and align, securely fasten and install free from distortion or defects.

E.	Deform threads of tail end of anchor studs after nuts have been tightened to prevent accidental removal and vandalism.

3.06	FIELD QUALITY CONTROL

Specification Guide Note: Use the following Paragraphs when manufacturer’s field services are desired to verify the quality of the installed components. Establish the number and duration of periodic site visits required by the Manufacturer and specify below. Consult with the Manufacturer for services required. Delete if field services are not required.

A.	When necessary have the manufacture assist in installation.

B.	Manufacturer’s Field Services: Have manufacturer’s technical representative schedule site visits to review work as follows:

1.	After delivery and storage of products.

2.	When preparatory work for which work of this Section depends is complete, but before installation begins.

3.	[Weekly] [2 times] during progress of work [at [25%] and [60%]] of completion.

4. 	Upon completion of work, after cleaning is carried out.

C.	Testing: Test on site 100% of anchors relying upon chemical adhesive fasteners using load cell test apparatus in accordance with manufacturer's written recommendations.

3.07	ADJUSTMENT

A.	Lubricate moving parts to operate smoothly and fit accurately.

B.	Complete "Initial Inspection - Certification for Use" form included in Equipment Manual and Inspection Log Book provided by manufacturer.

3.08	FINAL CLEANING

A.	Do cleanup in accordance with Section [01 74 00 - Cleaning and Waste Management].

B.	Upon completion, remove surplus and excess materials, rubbish, tools and equipment.

3.09	PROTECTION

A.	Protect installed product from damage during construction in accordance with Section [01 76 00 - Protecting Installed Construction].

B.	Make good damage to adjacent materials caused by window washing equipment installation.

3.10	MAINTENANCE

A.	Use only standard parts of product line of manufacturer of window washing equipment.

B.	Perform work during regular trade working hours satisfactory to [Owner] [Architect] [Consultant].

C.	Provide emergency call-back at no extra cost and ensure fulfillment of maintenance and emergency service without undue loss of time to [Owner] [Architect] [Consultant].

D.	Ensure that maintenance personnel register with designated building personnel at time of inspections and maintenance.

END OF SECTION